Intro to SoC Design Course

LAB 08

API and Final Application

Issue 1.0

- Contents

1	Introduction	1

1.1	Lab overview	1

1.1.1	Software programming:	1

1.1.2	Demonstrate the SoC:	1

2	Learning Objectives	1

3	Requirements	2

4	Provided files	2

5	Software	3

5.1	API Development	3

5.1.1	Create API file	3

5.1.2	Add the retarget file	3

5.1.3	File structure	4

5.2	Application development	5

5.2.1	Application using polling	5

5.2.2	Application using interrupt (power saving)	5

5.2.3	Example of the demo:	6

6	Extension work	6

6.1	Extra tasks for this lab:	6

# Introduction

## Lab overview

In previous labs, we used the CMSIS and developed drivers for the peripherals. In this lab, we will develop an API that has more generic and easy-to-use functions. Then, based on the API, we will develop a final game application: Snake.

The work in this lab includes:

### Software programming:

- Using the functions provided by software drivers and CMSIS, develop an API that provides more generic and easy-to-use functions for application development.

### Demonstrate the SoC:

- Develop a final application (such as the Snake game) to demonstrate the SoC

- Use sleep mode to reduce the power consumption of your application

Figure 1: API and Application Development hierarchy

# Learning Objectives

- Modify reusable and easy to use APIs.

- Develop a single player snake game using C program which implements UART and timer interrupt handlers to control and display/ detect the snake’s coordinates.

# Requirements

This lab requires the following hardware and software:

- Hardware:

  - Diligent BASYS 3 FPGA board connected to computer via MicroUSB cable. A constraints file for this board is also provided.

  - VGA-compliant monitor and VGA cable to connect your board

- Software

  - Xilinx Vivado

  - Keil uVision

  - TeraTerm

# Provided files

You will need the files from the previous labs along with the following files which are provided with this Lab:

| File name | Description |
| --- | --- |
| core_cm0.h | CMSIS Cortex-M0 core peripheral access layer header file |
| core_cmFunc.h | CMSIS Cortex-M core function access header file |
| core_cmInstr.h | CMSIS Cortex-M core instruction access header file |
| cm0dsasm.s | Includes interrupt vectors and other setup assembly code |
| main.c | Includes the main program and interrupt service routines |
| EDK_CM0 | Defines the interrupt numbers and memory map etc. |

# Software

## API Development

### Create API file

Under the “Device” folder, create a head file called “API.h” and a C file called “API.c”.

Write the functions in “API.c” and include all the function calls in “API.h”.

Suggested functions are as follows:

| API Functions | Description |
| --- | --- |
| void SoC_init(void) | SoC initialization |
| void rectangle(int x1,int y1,int x2,int y2, int color) | Draw a rectangle on the screen. |
| void clear_screen (void) | Clean up the screen. |
| int read_switch | Read the value of the 8-bit switches. |
| write_LED | Write a value to the 8-bit LEDs. |
| void Display_Int_Times (void) | Display the number of interrupts that occurred using the 7-segment display. |
| void delay(int value) | Software delay program |
| char random (char min, char max) | A simple random generator based on system tick |

### Add the retarget file

The retarget file allows us to use print library functions such as “printf()”. To add the retarget file:

- Add the “retarget.c” file to the “Device” folder.

- Implement the retarget functions, for example:

| Retarget Functions | Description |
| --- | --- |
| int KBHIT(void) | Wait for keyboard hit. |
| int fputc(int ch, FILE *f) | Input characters |
| int fgetc(FILE *f) | Output characters |
| unsigned char VGAPutc(unsigned char my_ch) | Output characters to VGA |
| unsigned char UartPutc(unsigned char my_ch) | Output characters to UART |
| unsigned char UartGetc(void) | Input characters from UART |

Example code:

//define UartPutc

unsigned char UartPutc(unsigned char my_ch)

{

UART->DATA=my_ch;

return (my_ch);

}

//define fputc

int fputc(int ch, FILE *f) {

return (UARTPutc(ch));

}

//use printf in main.c

printf("HelloWorld");

### File structure

The files can be organized as follows:

Core folder

Device folder

Application folder

core_cm0.h

cm0dsasm.s

main.c

core_cmFunc.h

core_cmInstr.h

EDK_CM0.h

edk_driver.c

edk_driver.h

edk_api.c

edk_api.h

retarget.c

## Application development

The following ideas can be used to program the Snake game:

### Application using polling

- Main program

  - Initialize the SoC.

  - Initialize the game.

  - Repeat the following:

    - Check if keyboard hits; if yes, then,

      - Update snake direction.

    - Move the snake.

    - Check if it hits the wall; if yes, then,

      - Game over

    - Delay for a short time.

### Application using interrupt (power saving)

- Main program

  - Initialize the SoC.

  - Initialize the game.

  - Enter the sleep-on-exit mode.

- Timer interrupt handler

  - Trigger the snake to move one step.

  - Detect if the target is reached or if the snake hits the wall.

- UART interrupt handler

  - Input the command from the keyboard.

  - Change the direction of the snake.

Interrupt-driven Mode

### Example of the demo:

Demo Example

# Extension work

## Extra tasks for this lab:

- Use a sampling energy meter (or other equipment) to measure the run-time power consumption of your game application.

- Optimize your code to reduce power consumption.

- Explore other games, such as TERIS, PACMAN, BREAK, TICTAC, etc.