Efficient Embedded Course

LAB 9

TRUSTZONE EXERCISE:

SECURE AND NON-SECURE BLINK

Issue 1.0

- Contents

1	Introduction	1

1.1	Lab overview	1

2	Learning Objectives	2

3	Requirements	2

3.1	Software requirements	2

3.2	Hardware requirements	2

4	Setting up the project	3

4.1	Partitioning the memory into secure and non-secure banks using STM32Cube Programmer	3

4.2	Creating the project using STM32Cube MX	5

5	Programming in Keil	7

5.1	Project_S	8

5.1.1	Define starting address in main.c	8

5.2	Project_NS	9

5.2.1	Configuring the settings for Project_NS	10

5.3	Back to Project_S	12

5.3.1	Configuring the settings for Project_S	12

5.4	Building and loading the project	13

5.5	Running and understanding the program	14

5.5.1	Running Project_NS	14

5.5.2	Running Project_S	15

# Introduction

## Lab overview

In this lab, you will implemente a project utilising Arm’s TrustZone feature. TrustZone is a special feature available in newer Arm Cortex M microcontrollers such as the Arm Cortex M33  in the STM32 Nucleo-L552ZEQ board used in this lab. TrustZone enables the user to partition the board ‘Secure’ and ‘Non-Secure’ memory banks and use the memory bank to run different parts of a program on each bank. This reduces the potential of unwanted breaches of sensitive data/program features by isolating them from the rest of the program.

When TrustZone is enabled, program elements from the ‘Secure’ partition are able to call program elements (i.e. function calls, global variables) from the ‘Non-Secure’ partition; However, elements from the ‘Non-Secure’ partition are unable to access program elements from the ‘Secure’ partition.

In this lab, we will test these concept using a simple program that assigns LEDs ‘Secure’ and ‘Non-Secure’ modes and calling them to blink from both the ‘Secure’ and ‘Non-Secure’ memory banks and observe what happens.

The lab is divided into two parts. In Part 1, we will use the tools (STM Programmer and STM CubeMx) provided by STM Microelectronics to do the following:

- Partition the memory into two banks – i.e. secure and non-secure memory banks using STM32Cube Programmer

- Configure the Nucleo-L552ZEQ board using the STM32Cube Programmer

- Set up the project using the STM32Cube MX

- Export the program with all the settings to KEIL MDK using STM32Cube MX

In Part 2, we will use KEIL MDK to:

- Program the board

- Observe the result on the debug window

Fig 1: Arm TrustZone project summary

# Learning Objectives

- Partitioning the memory into secure and non-secure banks

- Create secure and non-secure world projects and configure their settings

- Assign security levels to IO peripherals

- Run simple program to call function from secure world to non-secure world

# Requirements

## Software requirements

- Keil µVision5 MDK IDE

  - Please check the Getting Started with Keil guide on how to download and install it.

- STM32Cube Programmer

  - For more information and download, click here.

- STM32Cube MX

  - For more information and download, click here.

## Hardware requirements

- STM32 Nucleo-L552ZE-Q board

  - For more information, click here.

# Setting up the project

## Partitioning the memory into Secure and Non-secure banks using STM32Cube Programmer

- Connect the board to your PC and open the STM32Cube Programmer

- Select the ST-LINK option from the drop-down menu and click connect

- Click the OB (Option Bytes) tab on the left. TrustZone is disabled at default, so we need to enable it.

1

2

3

Fig 2: STM32Cube Programmer

- Click User Configuration, and check DBANK to enable Dual Bank mode. This will allow us to partition the memory in 2 banks - one Secure bank, and one Non-secure bank. Click Apply once you have picked the correct options.

Fig 3: Option Bytes Configuration

- Under Secure Area 1 and Secure Area 2, we need to define the start and the end address of the two banks. In the project, the addresses are chosen as below. Once done, click apply.

- In this lab, Area 1 will be used as the ‘Secure’ memory bank while Area 2 will be used for the ‘Non-secure’ memory bank.

Fig 4: Memory Address Configuration

- Click Disconnect.

## Creating the project using STM32Cube MX

- Open STM32Cube MX and click ‘ACCESS TO BOARD SELECTOR’.

Fig 5: STM32Cube MX Home page

- Under the ‘Board Selector’ tab, search for ‘NUCLEO-L552ZE-Q’. Select the board and click ‘Start Project’.

Fig 6: Board selector settings

- If asked ‘Initialize all peripherals with their default Mode?’, click ‘Yes’.

- If asked ‘Do you want to create a new project :’, select the ‘with TrustZone activated ?’ option and click ‘OK’.

- For this project, we are using three on board LEDs, green Led (PC7) for Secure world, blue Led (PB7) and red Led (PA9) for Non-secure world.

Fig 7: Pinout view

- Under system core, select GPIO, and change Pin Context Assignment for all the 3 LED pins - ‘Cortex-M33 Secure’ for the green LED pin and ‘Cortex-M33 Non-secure’ for the red and blue LED pins.

- This means that the green LED can only be accessed by program elements in the ‘Secure’ memory bank while the blue and red LEDs can be accessed from both the ‘Secure’ and ‘Non-secure’ memory banks.

Fig 8: GPIO settings

- Navigate to the ‘Project Manager’ tab. Under the ‘Project’ section, select ‘MDK-ARM’ under the ‘Toolchain / IDE’ setting. Name the project and its location as you please.

Fig 9: Project Manager settings

- Click ‘Generate Code’.

- Open the project with KEIL MDK.

# Programming in Keil

In Keil, notice that the project is divided into two projects – project_S and project_NS, corresponding to the program in the Secure and Non-secure memory banks respectively. (Note that project is just a place holder for the actual name of the project)

Fig 10: Project overview in Keil

## Project_S

### Define starting address in main.c

In ‘main.c’ file under project_S, make sure to define the correct starting address for the Non-secure memory bank.

Fig 11: Non-secure starting address

Notice the function that jumps from the Secure bank to the Non-secure bank as seen in the NonSecure_Init function in line 129.

Fig 12: ‘NonSecure_Init’ function

## Project_NS

The main.c file in Project_NS is for the Non-secure bank. Here the blue and red LEDs are toggled at a certain rate.

Fig 12: None-secure ‘main.c’ file

- Right-click on ‘Project project_ns’ and ‘Set as Active Project’.

- Click the ‘Options for Target…’ button.

Fig 13: ‘Option for Target…’ button

### Configuring the settings for Project_NS

- Make sure the IROM1 and IRAM1 addresses are the same as below.

- Notice how the address is covered by the addresses that was defined for ‘Secure Area 2’ in the STM32Cube Programmer is section 4.1.

Fig 14: IROM1 and IRAM1 Memory Areas

- Navigate to the ‘Debug’ tab and select the ‘ST-Link Debugger’. Then, click ‘Settings’.

Fig 15: Debug configurations

- Navigate to the ‘Flash Download’ tab.

- Make sure to add the following ‘On-chip Flash’ and the correct ‘RAM for Algorithm’ start address and size. Click ‘OK’ and ‘OK’ again.

- Again notice how the memory locations are covered by the addresses that were defined for ‘Secure Area 2’ in the STM32Cube Programmer is section 4.1.

Fig 16: Flash download configurations

## Back to Project_S

- Now set project_s as the active project and click the ‘Options for Target…’ button.

### Configuring the settings for Project_S

- Configure IROM2 and IRAM2 as follows.

Fig 17: IROM2 and IRAM2 Memory Areas for the Secure bank

- Navigate to the ‘Debug’ tab and select ‘ST-Link Debugger’. Then, click ‘Settings’.

Fig 18: Debug configurations

- Make sure to set the following settings. Once finished, navigate to the ‘Flash Download’ tab.

Fig 19: Debug window

- Make sure to add the following ‘On-chip Flash’ and the correct ‘RAM for Algorithm’ start address and size. Click ‘OK’ and ‘OK’ again.

Fig 20: Flash download configurations

## Building and loading the project

- Make project_NS as the active project.

- Under the ‘Projects’ tab, click Batch Setup and build both projects.

Fig 21: Batch Setup

- Click the ‘Load’ button to flash the board.

## Running and understanding the program

### Running Project_NS

- Click the ‘Start/Stop Debug Session’ button.

Fig 22: Debug window

- Click the ‘Run’ button.

- You will notice that only the blue and red LEDs will start to blink but not the green LED. This is because the green LED can only be toggled from the ‘Secure’ memory bank.

### Running Project_S

- Exit the Debug Session by clicking on the ‘Start/Stop Debug Session’ button again.

- Set project_S as the active project.

- Batch rebuild the project.

- Click the ‘Start/Stop Debug Session’ button to enter the Debug Session.

- Click the ‘Run’ button to run the program.

- Once you clicked ‘Run’, you notice that all three LEDs are now blinking. This is because the green LED is now being toggled by part of the program in the ‘Secure’ memory partition.