VLSI Fundamentals: A Practical Approach

Labs Getting Started Guide

Issue 1.0

- Contents

1	Introduction	1

1.1	Course lab overview	1

1.2	Scope of this guide	1

2	Microprocessor	2

2.1	Architecture	2

Test Program	4

2.2	Microarchitecture	4

2.3	Physical Design	8

2.4	SystemVerilog	10

3	Hardware and Software Requirements	10

3.1	Overview	10

3.2	Digital VLSI Chip Design with Cadence and Synopsys CAD Tools	11

3.3	Cadence Tools	11

3.4	Synopsys Tools	12

4	Lab Files	12

5	Troubleshooting	12

# Introduction

## Course lab overview

This course contains four labs in which you will learn to design a digital integrated circuit using commercial CAD tools from Cadence and Synopsys.  Specifically, you will learn to design custom and synthesized parts of a simple microprocessor in an AMI 0.6 mm CMOS process.  The labs provide an 8-bit microprocessor missing one of each interesting type of component.

At the end of the lab students will be able to:

- Design, implement, simulate and verify leaf cells and simple logic gates from transistor level schematic to layout.

- Perform simple checks such as design rule checking (DRC), and layout-vs-schematic (LVS) verification on leaf cells, logic gates and logic blocks layouts.

- Use NC-Verilog to simulate and verify the operation of logic blocks.

- Use design compiler to synthesize logic gates from hardware description language and use SOC Encounter to place and route logic design.

- Assemble a chip from schematic, layout, add pad frame and then tape out in GDSII format.

As prerequisite to the labs, students should have completed an introductory course on digital design. An introductory course on computer architecture and the ability to read SystemVerilog will make the labs more satisfying but are not essential. After Lectures 1 and 2, a student should be able to complete Lab 1.  After Lecture 3, a student should be able to complete the other labs.

After completing the labs, students should be ready to independently design a chip at the scale of a class project.

## Scope of this guide

This Getting Started Guide is intended to give an overview of the simple microprocessor that will be built, to outline the hardware and software requirements for the laboratories, and to describe how to install the files specific to these labs. The microprocessor section is of interest to both students and instructors, while the remaining sections are primarily of interest to the instructor.

# Microprocessor

This section describes the simple microprocessor implemented in the labs.  This section describes the architecture and microarchitecture of the processor.  The context should make the labs more rewarding but is not essential to complete the labs.

The processor is based on the multicycle microprocessor described in Digital Design and Computer Architecture, by Harris and Harris.  It supports a subset of the ARMv7 instruction set described below.  To fit the processor on a small chip using a legacy technology node, the datapath is 8 bits and the number of registers are 8.

The multicycle microarchitecture was selected because of:

- Small hardware requirement.

- Supports a single memory bus-to-shared instruction and data memory.

- The instructions are 32 bits; therefore, the instruction register comprises four 8-bit registers loaded over four cycles of instruction fetch.

## Architecture

The microprocessor supports the following instructions:

Data Processing (with register and immediate second sources)

- ADD, SUB, AND, OR

Memory (with positive immediate offset)

- LDR and STR

Branch

- B

The data processing instructions are available in a form that set the condition code flags, and each instruction is available in conditional form.

Because the number of registers are 8, only the bottom 3 bits of each register address are used.  The program counter is R7.  Reading the program counter gives 8 bytes beyond the address of the current instruction.

Each instruction is encoded with a 32-bit machine language word. The encoding for each type of instruction are given in Figure 1.

In Figure 1, the Data-processing 32-bit encoded machine language word contains six fields which are cond, op, funct, Rn, Rd and Src2. The fields cond, op and funct indicate the operation to be performed by the instruction.

Figure 1: Machine Language Encodings

Each instruction has a 4-bit condition field that determines whether the instruction should be executed based on the value of the condition flags or status flags.  The conditions are given in Table 1.

Table 1: Condition Codes

| Cond | Mnemonic | Name | Codes |
| --- | --- | --- | --- |
| 0000 | EQ | Equal | Z |
| 0001 | NE | Not Equal | Z |
| 0010 | CS | Carry Set | C |
| 0011 | CC | Carry Clear | C |
| 1000 | HI | Unsigned Higher | CZ |
| 1001 | LS | Unsigned Lower/Same | CZ |
| 1100 | AL | Always |  |

The funct field is further broken down into I, cmd and S. These subfields are described below.

- I:

  - This bit is 1 when Src2 is an immediate (imm8). Data processing instructions accept an 8-bit immediate as the second source when I=1.

  - This bit is 0 when Src2 is a register Rm.

- cmd: This is a 4-bit command field that indicates the type of data processing instruction. The codes are 0100 for ADD, 0010 for SUB, 0000 for AND, and 1100 for OR.

- S: This bit indicates when the instruction sets the condition flags. The arithmetic/logic unit (ALU) produces two condition code bits, Z and C, indicating that the result is zero, or has a carry out.  Data-processing instructions with S=1 set the condition code register based on codes from the ALU.

Rn and Rm are the first and second source registers.  Rd is the destination register (or the value to write for a STR).

Memory instructions always add a 12-bit immediate (imm12) offset to the base address specified in Rn.

Memory instructions accept a L field.  1 indicates LDR and 0 indicates STR.

Branches multiply the signed 2’s complement immediate by 4 and add them to PC + 8.

## Test Program

The test program in Snippet 1 exercises all the instructions and uses the condition codes.  Both assembly and machine language are shown. If it runs correctly, it will write the value 7 to the address 100. This test program will be used in testing the processor at the end of the lab.

Snippet 1: Test Program

## Microarchitecture

Figure 2 shows the partitioning of the microprocessor chip into a datapath, a controller, and an aludecoder block.  The partitioning reflects the design styles taught in the labs.

The datapath is made of 8-bit wordslices. The controller is synthesized from a SystemVerilog specification into logic gates and then placed and routed automatically.  The aludecoder block, which could have been synthesized as part of the controller, is instead pulled out so that students can design a simple random logic by hand.

The chip uses 2-phase nonoverlapping clocks, ph1 and ph2. The 2-phase clocks are used to avoid hold time issues. The system is a synchronous system which is triggered by the rising edge of ph1, however; the memory is written during the second half of the cycle (ph2) when that the address and data are stable.

The chip also has a reset input which initializes the program counter to 0, clears the condition codes, and directs the main FSM to begin fetching the first instruction.

The chip has an 8-bit interface to external memory with unidirectional ReadData and WriteData busses.

Figure 2: Simple Processor Chip Partitioning

Figure 3 shows the datapath, controller, and alucontrol, adapted from the multicycle processor in Digital Design and Computer Architecture, ARM Ed by Harris & Harris.  The heavy (black) lines are 8-bit busses, except Instr, which is a 32-bit signal coming from four 8-bit instruction registers.  Control signals are in blue, while datapath signals are black.  The memory is also shown in context with the datapath, although it is physically off-chip.

Figure 3: Microprocessor Multicycle Datapath, Controller, and ALUDecoder

Figure 4 shows the controller which is partitioned into a decoder and conditional logic.  The decoder computes the control signals based on the instruction which should be executed.  It has a main finite state machine that produces the appropriate control signals for each step, and small combinational logic blocks to control the program counter and signals that only depend on the op field of the nstruction.

CondEx, which is based on the condition codes and cond field, which is TRUE if the instruction should be conditionally executed. It kills the write enable signals to prevent it updating the architectural state when the instruction shouldn’t be executed.

Figure 5 shows the main finite state machine in the decoder. The first four states fetch the instruction one byte at a time. The appropriate IRWrite signal is asserted in each of these states to write fetched byte into the instruction register. In each fetch state, the program counter is incremented by one to point to the next byte in the instruction.

Figure 4: Microprocessor Controller

When in decode state, the ALU computes PC + 8, in case the program counter needs to be read, and the operands are read from the register file or sign extender. Then the main FSM transitions to an appropriate state depending on the type of instruction being executed.

LDR and STR both compute the address in the memory by adding the base address to the immediate offset.  LDR then reads the value from the memory and writes it to the register file.  STR stores the value to memory.

Data processing instructions perform the appropriate ALU instruction on one source from a register and a second source from either another register or the immediate value and then write the result to the register file.

Branch instructions add the branch offset to PC+8 and write the branch destination back to the program counter.  The FSM eventually returns to the S0: Fetch state to fetch the next instruction.

Figure 5: Main FSM

## Physical Design

Figure 6 shows a floorplan for the processor.  Observe the datapath in the center with eight identical bitslices.  A zipper at the top of the datapath contains the register file address decoders and the buffers to drive multiplexor selects and register enables.  The controller sits above the datapath.

Figure 6: Floorplan

Figure 7: Chip Layout

The entire processor is surrounded by a pad frame with large drivers to interface with the external physical pins of the chip.  The floorplan is annotated with dimensions. Figure 7 shows the layout of the entire chip, and Figure 8 shows a zoom into the datapath.  These show that the datapath has a regular structure of bitslices and wordslices, while the controller is made of rows of standard cells routed together with a mess of wires.

Figure 8:Datapath Layout

Figure 9 shows a slice plan for the datapath showing how each bitslice is routed horizontally. The slice plan is helpful for planning wires because there is finite space to run horizontal metal3 wiring tracks over the cells.  Each cell is annotated with its width, measured in wiring tracks (8 ), so it is easy to sum up the entire width of the datapath.

Figure 9: Slice Plan

## SystemVerilog

The processor is described in behavioral SystemVerilog in processor_multi.sv. This file is provided along with the laboratory files.  The top level comprises the microprocessor chip and the external memory.  The memory is initialized from a memfile.dat file containing the machine language instructions for the test program given in the code snippet above. A test bench generates the clock and reset signals for the top-level module and checks for a memory write to verify correct operation.

During the labs, you will verify individual components by generating a structural netlist for the component from the schematics and substituting it for the corresponding behavioural component in the SystemVerilog model.

# Hardware and Software Requirements

This section describes the hardware and software requirements for the lab, including an overview, Linux server requirements, and Cadence and Synopsys tool requirements.

## Overview

Setting up and maintaining commercial VLSI design tools requires a major investment of time and money well beyond that needed for CAD tools in most other fields.  It is most practical at a research university where the tools are already used and supported for research purposes, and where staff have the expertise and continuity to maintain the tools.  However, faculties can also acquire and install the tools themselves for the purpose of a CMOS VLSI course.

Acquiring the tools involves signing license and non-disclosure agreements.  Beware that these agreements may forbid students in online courses from using the tools remotely.

## Digital VLSI Chip Design with Cadence and Synopsys CAD Tools

Installing, configuring, and fully using the Cadence and Synopsys CAD tools is beyond the scope of this Getting Started Guide.

Please use the following links to download the required libraries to use the lab as is (that is call the commands as shown in the provided lab documents). You may need to edit the paths and environment variables to use these files.

Download the NCSU libraries (CDK v1.6.0-beta) from here. You need to first register with an email.

- https://eda.ncsu.edu/ncsu-cdk/

Download the UofU technology libraries, the version to download depends on your Cadence tool version, follow the instructions in this link:

- http://www.cs.utah.edu/~elb/cadbook/UofU/TechLib/index.html

Download and install the libraries listed here:

- http://www.cs.utah.edu/~elb/cadbook/index.html

  - UofU_TechLib_ami06 (you may have installed this in step 2 above)

  - UofU_Analog_Parts

  - UofU_Example

  - UofU_Digital_v1_2

  - UofU_Pads

Download the scripts in Appendix A using this link:

- http://www.cs.utah.edu/~elb/cadbook/Chapters/AppendixA/index.html

Download the scripts (.synopsys_dc_setup and syndc.tcl) in Appendix B using this link:

- http://www.cs.utah.edu/~elb/cadbook/Chapters/AppendixB/index.html

## Cadence Tools

The labs have been tested with:

- IC 6.1.8 (Virtuoso schematic and layout entry)

- ASSURA 615 (DRC, LVS, Extraction for old technologies)

- EDI 10.1 (placement and routing)

Note that Assura and EDI are not officially supported on RHEL7 but have been used successfully.

## Synopsys Tools

The labs have been tested with version syn_L-2016.03-SP4.

# Lab Files

The 2020 release of the lab files are in cmosvlsi.tar.gz, available to instructors who adopt this course from the Arm University Program.  Install them on your Linux server in a location such as

/courses/cmosvlsi/20

The administrator should create a user such as cmosvlsi to control permissions.  The lab files should be read-only by regular users, while the solutions should be readable only by the instructor.

sudo chown -R cmosvlsi /courses/cmosvlsi/20

sudo chmod -R 755 /courses/cmosvlsi/20

sudo chmod -R 700 /courses/cmosvlsi/20/lab_sol

If you choose to install in a different location, change the paths in /courses/cmosvlsi/20/cadence/cds.lib and let your students know about the new paths for the labs.

# Troubleshooting

Technical support for VLSI CAD tools is a perennial challenge.  Preferably, the faculty member teaching the course will have industrial experience using and troubleshooting the Cadence and Synopsys tools and a staff system administrator will have or can develop experience with licensing, tool installation, and Linux support.

The Cadence and Synopsys University Programs provide technical support related to software installation and licensing to one professor and to a system administrator, but not to students.